| К сожалению, русского текста дать не могу – под запретом | - Cable Network and Wire Installation CCTV cables are laid separately from the wiring of more than 60 V (in separatewire box). In a body of a tunnel cables are laid in a spiral wrap hose with a heat-shrink sleeve. To transfer a signal from video cameras to videoswitchboards and monitors cable РК 75-3-32 is applied. Cable ПВС 3x15 ensures the video cameras power supply. The distance between the wiring and CCTV cables of supply conductors and fixture wires should not be less than 0.5m if they are parallel. In case this wiring needs to be laid at the distance of 0.5m from supply conductors and fixture wires, they should have a protection from interference (should be laid in a separate metal tube or a spiral wrap hose). But it is allowed to reduce the distance between the wiring and CCTV cables of supply conductors and fixture wires up to 0.25m. The distance between cables and isolated wiring laid openly along the building structure items straight to the places of burnable materials storage should not be less than 0.6m. In case cables and wiring cross pipework, the distance between them should not be less than 50mm. |
| --- | --- |

| - ВСТУПЛЕНИЕ Гео-инженерное исследование строительной площадки действительного торгового центра было выполнено ЗАО 'НВ ТИЗИС', и полученные данные представлены в "Отчете о инженерно-геологических исследованиях проектируемой площади действительного торгового центра, микрорайон Дзержинского, Историческая улица д.175, Волгоградской области" № 16304 от 1998, Волгоград. В рамках инженерного-геологического исследования были пробурены 14 скважин на абсолютных отметках от 8 до 30 метров и была взята 21 проба из разведочных выработок с помощью конического пенетрометра. Отчет включает в себя 10 инженерно-геологических секций, свойства индекса почвы, результаты анализа мощности и деформационных характеристик грунтов, результаты химического анализа на определение коррозионной агрессивности грунтовых вод и пластов. Данный отчет был представлен в целях описания инженерно-геологического разреза, типов основ и их несущей способности. - ИНЖЕНЕРНО-ГЕОЛОГИЧЕСКИЙ РАЗРЕЗ Мощность неконтролируемого слоя заполнения варьируется от 0,5м до 2м. Под слоем заполнения суглинок IGE2 и суглинок песка IGE3 твердой консистенции, расположенные к западу от площади застройки, на осях координат Ж-Ж (на востоке) и З-З (на западе) с толщиной, постепенно увеличивающейся с востока на запад. Грунты IGE2 и IGE3 сужаются на глубине между 8м и 19м. К востоку от оси Ж-Ж, песок IGE4 средней плотности расположен на месте почв суглинка. Грунтовые воды ниже 35м не исследовались. 3. ТИПЫ ФУНДАМЕНТА Строительные основы – железобетонный отдельный фундаментУровень верхней отметки плиты - 156,8 м. Минимальное возвышение подошвы основы - 153.80 м. Под каждым слоем основы находится пласт тощего бетона толщиной 10 см. глубина утопленности основ - 3 м. При раскопке земля будет выкладываться на слой суглинка IGE3 к западу от области раскопки или на слой песка IGE4 в к востоку от области раскопки. Основание на плане главным образом квадратное, с размерами от 2,5м до 5,75м. Самое большое основание имеет прямоугольную форму с размерами 4х7,5м. Типы оснований и их размеры показаны на чертежах. | # 1. INTRODUCTION Geotechnical investigation of Real shopping center construction area was carried out by ZAO ‘NV TISIZ’ and their findings have been given in the “Report on The Engineering and Geological Investigations at The Projected Real Shoping Center Area, Dzerjinski Zone, Istoricheskoi str. No 175, of the Volgograd district’’ dated Volgograd 1998, report no: 16304. During the geotechnical survey 14 boreholes of 8 m -30 m were drilled, 21 static cone penetrometer tests were carried out. The report includes 1o engineering geological sections, soil index properties, strength and deformation characteristics test results of the soils, chemical test results for corrosion aggressiveness evaluation of the ground water and soils. This report has been reviewed for the purpose of describing the soil profile, the foundation types and their bearing capacity. # 2. SOIL PROFILE The thickness of the uncontrolled fill layer varies between 0.5 m and 2 m. The mean ground level is about 156m Beneath the fill layer, loam IGE2 and sand loam IGE3 of solid consistency are located at west of construction area, between the building axis G-G (east) and H-H (west); with thickness progressively increasing from east to west. The IGE2 and IGE3 soils have subsiding properties between the depths 8m and 19m. At east of the axis H-H, the IGE4 sand of medium density are located in lieu of loam soils. The groundwater has not been observed down to the depth of 35m. # 3. FOUNDATION TYPES Building foundations are reinforced concrete pad foundations. The slab on grade top level is 156,8 m. The bottom elevation of the foundations is at least 153.80 m. There is 10 cm thick lean concrete layer beneath each foundation. The embedment depth of the foundations is 3 m. All the foundation bases will be in excavation and will be laid on loam soils IGE3 at west of the area or on sand layer IGE4 at east of the area. The foundations are mostly square in plan with dimensions varying between 2.5 m to 5.75 m. The biggest foundations of rectangular shape have dimensions 4 m by 7.5 m. The foundation types and their dimensions in plan are shown on the drawings. |
| --- | --- |