| Перевод инструкции по Caterpillar – компания производит землеройно-транспортную технику. |  |
| --- | --- |
| English | Russian |
| The left brake pedal controls the amount of brake oil pressure that is used to apply the service brakes. The left brake pedal also controls the amount of pressure that actuates the impeller clutch. The impeller clutch is positioned between the engine and the torque converter. By using the left brake pedal, the operator may divert engine power to the implement hydraulic circuit without putting the transmission in neutral. As the operator depresses the pedal, the impeller clutch pressure drops quickly to a working pressure. The pressure is then modulated to a reduced pressure. This sequence occurs during the next ten degrees of pedal travel. Depressing the left brake pedal past this point applies the service brakes. Typical Torque Converter Impeller Clutch X Approximate pedal position in degrees. Y Approximate percent of impeller clutch pressure. Range A indicates the released position. In this range, the brake pedal does not apply the service brakes. Range B indicates the first two degrees of pedal movement. In this range, the impeller clutch pressure is reduced to the maximum working pressure level. Range B is the approximate pressure range for preventing slippage of the impeller clutch. Range C occurs approximately within the next ten degrees of pedal travel. The impeller clutch pressure is modulated from the maximum working pressure to the minimum impeller clutch pressure. Range D indicates the percent of service brake pressure. Range E indicates the impeller clutch pressure. | Левая педаль тормоза контролирует количество масляного давления тормоза, которое используется для управления рабочими тормозами. Левая педаль также управляет количеством давления, благодаря чему запускается муфта крыльчатки. Муфта крыльчатки расположена между двигателем и гидротрансформатором. Используя левую педаль, машинист может направить мощность двигателя на устройство гидроконтура, не устанавливая коробку передач в нейтральное состояние. В то время как машинист нажимает на педаль, давление муфты крыльчатки быстро снижается до рабочего давления. Затем давление уменьшается до пониженного. Такая последовательность повторяется в течении последующих десяти степеней хода педали. Нажимая на левую педаль тормоза, при прохождении через следующую точку срабатывают рабочие тормоза. Типичная муфта крыльчатки гидротрансформатора X - Приблизительная позиция педали. Y - Приблизительный процент давления муфта крыльчатки. Диапазон A показывает положение расцепления. В этом положении, педаль тормоза не приводит в действие рабочие тормоза. Диапазон B показывает два первых положения хода педали. В этом положении, давление муфты крыльчатки уменьшается до максимального рабочего давления. Диапазон B это приблизительный диапазон давления для предотвращения проскальзывания муфты крыльчатки. Диапазон С совершается примерно в следующих десяти положениях хода педали. Давление муфты крыльчатки снижается с максимального рабочего до минимального давления муфты крыльчатки. Диапазон D показывает процент рабочего давления тормозов. Диапазон E показывает давление муфты крыльчатки. |